A Project Report on

IOT BASED UNLOCKING OF HOME AUTOMATION SYSTEM AND FACE DETECTION USING ESP32

Submitted in partial fulfillment of the requirement for the award of the degree of

# BACHELOR OF TECHNOLOGY

# In

# ELECTRONICS AND COMMUNICATION ENGINEERING

By

K. MANIKANTA                 G.M.L. BHAVANI                 BH. SRI MADHURI                       (18A81A0426)                   (18A81A0416)                      (18A81A0466)

M. PURNASRI                A. HEMANTH SREE SAI          T. SUDHEER BABU

(18A81A0433)                 (18A81A0402)                    (18A81A0454)

Under the Esteemed Guidance of

## Dr.M.KOTESWARARAO M.Tech, Ph.D,

Professor

DEPARTMENT OF ELECTRONICS & COMMUNICATION ENGINEERING

# SRI VASAVI ENGINEERING COLLEGE

(Autonomous	)

(Affiliated to JNTUK & Accredited by NAAC- ‘A’ Grade)

Pedatadepalli, Tadepalligudem, West Godavari, AP -534101 2018-2022

DEPARTMENT OF

ELECTRONICS AND COMMUNICATION ENGINEERING

CERTIFICATE

This  is  to  certify  that  the  project  report  entitled  “IOT  BASED  UNLOCKING  OF  HOME  AUTOMATION  SYSTEM  AND  FACE  DETECTION  USING  ESP32”submitted  by  K.MANIKANTA  (18A81A0426),  G.M.L.BHAVANI  (18A81A0416),  Bh.SRIMADHURI  (18A81A0466),  M.PURNASRI  (18A81A0433),  A.HEMANTH  SREE  SAI  (18A81A0402),  T.SUDHEER  BABU  (18A81A0454)  in  partial  fulfilment  of  the  requirements  for  award  of  the  Degree  of  Bachelor  of  Technology  in  Electronics  and  Communication  Engineering,  from  Sri  Vasavi  Engineering  College,  Tadepalligudem  affiliated  to  JNTU  Kakinada  and  NAAC  with  ‘A’  grade  is  a  record  of  bonafide  work  carried  out  by  them  under  my  guidance  and  supervision.

PROJECT  GUIDE	HEAD  OF  THE  DEPARTMENT

(Dr.M.  KOTESWARA  RAO)	(Sri  Dr.E.  Kusuma  Kumari)

EXTERNAL  EXAMINER

DECLARATION

We  hereby  declare  that  the  project  entitled  “IOT  BASED  UNLOCKING  OF  HOME  AUTOMATION  SYSTEM  AND  FACE  DETECTION  USING  ESP32  ”  is  submitted  in  partial  fulfillment  of  the  requirements  for  the  award  of  Bachelor  of  Technology  in  Electronics  and  Communication  Engineering  under  the  esteemed  supervision  of  Dr.M.  KOTESWARA  RAO  M.Tech,  Ph.D,  Professor,  Department  of  ECE.

This  a  record  of  work  carried  out  by  us  and  results  embodied  in  this  project  report  have  not  been  submitted  to  any  other  university  for  the  award  of  any  degree.

## Project  associates

K.MANIKANTA                 (18A81A0426)

G.M.L.BHAVANI               (18A81A0416)

BH.SRI  MADHURI            (18A81A0466)

M.PURNASRI                     (18A81A0433)

A.HEMANTH  SREE  SAI  (18A81A0402)

T.SUDHEER  BABU           (18A81A0454)

ACKNOWLEDGEMENT

We  take  this  opportunity  to  express  our  profound  sense  of  gratitude  in  all  its  humbleness  to  our  beloved  Project  Guide,  Dr.M.KOTESWARA  RAO,  M.Tech,  Ph.D  Professor  in  Electronics  and  Communication  Engineering,  Sri  Vasavi  Engineering  College,  Tadepalligudem.  For  his  excellent  guidance,  meticulous  care  and  enthusiastic  encouragement  in  motivating  us  to  take  up  this  challenging  task  and  helped  us  in  the  completion  of  the  project.

We  would  like  to  express  our  gratitude  to  Project  Coordinator  Dr.PURNIMA  K  SHARMA,  M.  Tech,  Associate  Professor  of  our  college  for  the  excellent  support  and  encouragement  throughout  this  project.

We  are  indebted  to  Dr.E.KUSUMA  KUMARI,  Ph.D.,  Professor  &  Head  of  the  Department  of  Electronics  and  Communication  Engineering,  for  her  constant  encouragement  during  execution  of  the  project.

We  express  our  deep  sense  of  gratitude  to  Dr.G.V.N.S.R.  RATNAKAR  RAO,  Principal,    and  Management  of  Sri  Vasavi  Engineering  College  for  providing  necessary  facilities  to  carry  out  our  project  work  successfully.

We  would  like  thank  to  our  Teaching  and  Nonteaching  Staff  of  Department  of  ECE,  who  have  been  directly  and  indirectly  part  of  this  journey,  for  their  encouragement  to  complete  our  project  work.

An  Endeavour  over  long  period  can  also  be  successful  by  constant  effort  and  encouragement.  We  wish  to  take  this  opportunity  to  express  our  deep  gratitude  to  all  the  people  who  have  extended  their  cooperation  in  various  ways  during  our  project  work.  It  is  our  pleasure  to  acknowledge  the  help  of  all  those  respected  individuals.

## Project  associates

K.MANIKANTA                (18A81A0426)

G.M.L.  BHAVANI             (18A81A0416)

BH.SRI  MADHURI            (18A81A0466)

M.PURNASRI                      (18A81A0433)

A.HEMANTH  SREE  SAI  (18A81A0402)

T.SUDHEER  BABU           (18A81A0454)

INDEX

CONTENTS                                                                      Page.No

Abstract                                                                                    i

CHAPTER-1:  INTRODUCTION                                          1  -  7

1.1  Embedded  System  Implementation……………………..1

1.2  Micro  Processor  vs  Micro  Controller  ………………...3

1.3  Embedded  System  Software  …………………………...4

1.4  Applications  of  Embedded  Systems…………………....6

1.5  Implementation  Flow  …………………………………...6

CHAPTER-2:  2.1  LITERATURE  SURVEY                    7  -  8

CHAPTER-3:  WORKING  DESCRIPTION                      9  -  34

3.1  Existing  system…………………………………………..9

3.2  Proposed  system………………………………………....9

3.3  Nodemcu  ………………………………………………..10

3.4  Power  Supply…………………………………………...14

3.5  Transformer……………………………………………...15

3.6  Rectifier……………………………………………….....16

3.7  Capacitor…………………………………………………17

3.8  Voltage  Regulator……………………………………….17

3.9  ESP32  AI  Thinker  Cam………………………………..18

3.10  Relay……………………………………………………26

3.10.1  Relay  basics………………………………………….29

3.10.2  Energized  Relay  (ON)……………………………….30

3.10.3  De-Energized  Relay  (OFF)………………………….31

3.11  Pole  and  Throw………………………………………..32

3.11.1  Relay  Applications………………………………….33

3.11.2  Applications  of  Overload  Relay…………………...33

3.11.3  Relay  Selection……………………………………...34

CHAPTER-4:  SOFTWARE  REQUIREMENT                35 - 41

4.1  Adafruit  cloud  server…………………………………….35

4.2  Adafruit  IO  Setup………………………………………...36

4.3  IFTTT………………………………………………………41

CHAPTER-5:  RESULTS                                                     46 - 47

CHAPTER-6:  CONCLUSION  AND  FUTURE  SCOPE

6.1  Conclusion  ………………………………………………46

CHAPTER-7:-REFERENCES                                              47  -  48

ABSTRACT

In  this  fast  paced  life  of  21stcentury,  automation  plays  a  key  role  in  human  life.  Home  automation  allows  us  to  control  household  electrical  appliances  like  light,  door,  fan,  AC  etc.  It  also  provides  home  security  and  emergency  system  to  be  activated.  Home  automation  not  only  refers  to  reduce  human  efforts  but  also  energy  efficiency  and  time  saving.  The  main  objective  of  home  automation  and  security  is  to  help  handicapped  and  old  aged  people  who  will  enable  them  to  control  home  appliances  and  alert  them  in  critical  situations.

This  project  put  forwards  the  implementation  of  home  automation  and  security  system  using  NodeMCU  and  ESP32  Camera.  Home  appliances  are  connected  to  the  microprocessor  and  communication  is  established  between  the  NodeMCU  and  Android  mobile  device  or  tablet  via  Cloud.  This  project  would  develop  an  authentication  to  the  system  for  authorized  person  to  access  home  appliances.

## CHAPTER  1

## INTRODUCTION

Today’s  homes  require  sophistication  control  in  its  different  gadgets  which  are  basically  electronic  appliances.  This  has  revolutionized  the  area  of  home  automation  with  respect  to  an  increased  level  of  affordability  and  simplicity  through  the  integration  of  home  appliances  with  smart  phone  and  tablet  connectivity.  Smart  phones  are  already  feature-perfect  and  can  be  made  to  communicate  to  any  other  devices  in  an  ad  hoc  network  with  a  connectivity  options  like  cloud  server.  With  the  advent  of  mobile  phones,  Mobile  applications  development  has  seen  a  major  outbreak.

1.1  Embedded  system  implementation

Introduction:

An  embedded  system  is  one  kind  of  a  computer  system  mainly  designed  to  perform  several  tasks  like  to  access,  process,  and  store  and  also  control  the  data  in  various  electronics-based  systems.  Embedded  systems  are  a  combination  of  hardware  and  software  where  software  is  usually  known  as  firmware  that  is  embedded  into  the  hardware.  One  of  its  most  important  characteristics  of  these  systems  is,  it  gives  the  o/p  within  the  time  limits.  Embedded  systems  support  to  make  the  work  more  perfect  and  convenient.  So,  we  frequently  use  embedded  systems  in  simple  and  complex  devices  too.  The  applications  of  embedded  systems  mainly  involve  in  our  real  life  for  several  devices  like  microwave,  calculators,  TV  remote  control,  home  security  and  neighborhood  traffic  control  systems,  etc.

User  interface

Embedded system

Software

Hardware

Inputs

Outputs

Link  to  other  systems

Fig:  Overview  of  embedded  system

Embedded  system:

Embedded  system  includes  mainly  two  sections,  they  are

1.  Hardware

2.  Software

Embedded  System  Hardware:

As  with  any  electronic  system,  an  embedded  system  requires  a  hardware  platform  on  which  it  performs  the  operation.  Embedded  system  hardware  is  built  with  a  microprocessor  or  microcontroller.  The  embedded  system  hardware  has  elements  like  input  output  (I/O)  interfaces,  user  interface,  memory  and  the  display.  Usually,  an  embedded  system  consists  of:

- Power  Supply

- Processor

- Memory

- Timers

- Serial  communication  ports

- Output/Output  circuits

- System  application  specific  circuits

Embedded  systems  use  different  processors  for  its  desired  operation.  Some  of  the  processors  used  are

1.  Microprocessor

2.  Microcontroller

3.  Digital  signal  processor

1.2  Micro  Processor  vs.  Micro  Controller

Microprocessor

- CPU  on  a  chip.

- We  can  attach  required  amount  of  ROM,  RAM  and  I/O  ports.

- Expensive  due  to  external  peripherals.

- Large  in  size

- general-purpose

Microcontroller

- Computer  on  a  chip

- fixed  amount  of  on-chip  ROM,  RAM,  I/O  ports

- Low  cost.

- Compact  in  size.

- Specific  –purpose

1.3 Embedded  System  Software:

The  embedded  system  software  is  written  to  perform  a  specific  function.  It  is  typically  written  in  a  high  level  format  and  then  compiled  down  to  provide  code  that  can  be  lodged  within  a  non-volatile  memory  within  the  hardware.  An  embedded  system  software  is  designed  to  keep  in  view  of  the  three  limits:

- Availability  of  system  memory

- Availability  of  processor’s  speed

- When  the  system  runs  continuously,  there  is  a  need  to  limit  power  dissipation  for  events  like  stop,  run  and  wake  up.

Bringing  software  and  hardware  together  for  embedded  system:

To  make  software  to  work  with  embedded  systems  we  need  to  bring  software  and  hardware  together  .for  this  purpose  we  need  to  burn  our  source  code  into  microprocessor  or  microcontroller  which  is  a  hardware  component  and  which  takes  care  of  all  operations  to  be  done  by  embedded  system  according  to  our  code.

Generally  we  write  source  codes  for  embedded  systems  in  assembly  language,  but  the  processors  run  only  executable  files.  The  process  of  converting  the  source  code  representation  of  your  embedded  software  into  an  executable  binary  image  involves  three  distinct  steps:

- Each  of  the  source  files  must  be  compiled  or  assembled  into  an  object  file.

- All  of  the  object  files  that  result  from  the  first  step  must  be  linked  together  to  produce  a  single  object  file,  called  the  re-locatable  program.

- Physical  memory  addresses  must  be  assigned  to  the  relative  offsets  within  the  re-locatable  program  in  a  process  called  relocation.

The  result  of  the  final  step  is  a  file  containing  an  executable  binary  image  that  is  ready  to  run  on  the  embedded  system.

Source code

Assembler

Locator

Executable file

Linker

Processor

Flow  of  burning  source  code  to  processor

1.4  Applications:

Embedded  systems  have  different  applications.  A  few  select  applications  of  embedded  systems  are  smart  cards,  telecommunications,  satellites,  missiles,  digital  consumer  electronics,  computer  networking,  etc.

Embedded  Systems  in  Automobiles

- Motor  Control  System

- Engine  or  Body  Safety

- Robotics  in  Assembly  Line

- Mobile  and  E-Com  Access

Embedded  systems  in  Telecommunications

- Mobile  computing

- Networking

- Wireless  Communications

Embedded  Systems  in  Smart  Cards

- Banking

- Telephone

- Security  Systems

1.5  Implementation  flow:

Stage  1:

Considering  the  problems  of  existing  methods  and  giving  solution  to  that  problem  by  considering  the  basic  requirements  for  our  proposed  system

Stage  2:

Considering  the  hardware  requirement  for  the  proposed  system

For  this  we  need  to  select  the  below  components:

1.  Microcontroller

2.  Inputs  for  the  proposed  system  (ex:  sensors,  drivers  etc..,)

3.  Outputs  (ex:  relays,  loads)

Stage  3:

After  considering  hardware  requirements,  now  we  need  to  check  out  the  software  requirements.  Based  on  the  microcontroller  we  select  there  exists  different  software  for  coding,  compiling,  debugging.  we  need  to  write  source  code  for  that  proposed  system  based  on  our  requirements  and  compile,  debug  the  code  in  that  software  .

After  completing  all  the  requirements  of  software  and  hardware  we  need  to  bring  both  together  to  work  our  system.  For  this  we  need  to  burn  our  source  code  into  microcontroller,  after  burning  our  source  code  to  microcontroller  then  connect  all  input  and  output  modules  as  per  our  requirement.

## CHAPTER  2

## LITERATURE  SURVEY

Zigbee  based  home  automation  system  using  cell  phones:  To  monitor  and  control  the  home  appliances  the  system  is  designed  and  implemented  using  Zigbee.  The  device  performance  is  record  and  store  by  network  coordinators.  For  this  the  Wi-Fi  network  is  used,  which  uses  the  four  switch  port  standard  wireless  ADSL  modern  router.  The  network  SSID  and  security  Wi-Fi  parameter  are  preconfigured.  The  message  for  security  purpose  first  process  by  the  virtual  home  algorithm  and  when  it  is  declared  safe  it  is  re-encrypted  and  forward  to  the  real  network  device  of  the  home.  Over  Zigbee  network,  Zigbee  controller  sent  messages  to  the  end.  The  safety  and  security  of  all  messages  that  are  received  by  the  virtual  home  algorithm.  To  reduce  the  expense  of  the  system  and  the  intrusiveness  of  respective  installation  of  the  system  Zigbee  communication  is  helpful.

GSM  based  home  automation  system  using  cell  phones:  Because  of  the  mobile  phone  and  GSM  technology,  the  GSM  based  home  automation  is  lure  to  research.  The  SMS  based  home  automation,  GPRS  based  home  automation  and  dual  tone  multi  frequency  (DTMF)  based  home  automation,  these  options  we  considered  mainly  for  communication  in  GSM.  In  figure  shows  the  logical  diagram  the  work  of  A.  Alheraish,  it  shows  how  the  home  sensors  and  devices  interact  with  the  home  network  and  communicates  through  GSM  and  SIM  (subscriber  identity  module).  The  system  use  transducer  which  convert  machine  function  into  electrical  signals  which  goes  into  microcontroller.  The  sensors  of  system  convert  the  physical  qualities  like  sound,  temperature  and  humidity  into  some  other  quantity  like  voltage.  The  microcontroller  analysis  all  signal  and  convert  them  into  command  to  understand  by  GSM  module.  Select  appropriate  communication  method  among  SMS,  GPRS  and  DTFC  based  on  the  command  which  received  GSM  module.

## CHAPTER  3

## WORKING  DESCRIPTION

3.1  EXISTING  SYSTEM

In  the  existing  system,  home  appliances  are  controlled  through  Zigbee  and  other  devices.  But  they  are  limited  to  certain  area.  In  the  proposed  system  we  have  designed  so  as  to  control  home  appliances  from  anywhere  in  the  world.

DTMF  can  also  be  used  but  it  won’t  come  for  practically  and  also  not  reliable.

Drawbacks:

- Long  distance  communication  is  not  done

- No  Android  application  is  involved  so  we  cannot  operate  from  long  ranges.

3.2  PROPOSED  SYSTEM

In  this  project  we  are  using  NodeMCU  as  the  main  microcontroller  and  that  is  connected  to  cloud  server  which  is  Adafruit  IO.  Through  that  home  appliances  will  be  controlled.  Here  we  are  implementing  face  recognition  also  in  order  to  know  whether  the  person  is  authorized  or  not.

Block  Diagram:

3.3  NodeMCU:

Introduction  to  NodeMCU

NodeMCU  is  an  open-source  firmware  and  development  kit  that  plays  a  vital  role  in  designing  your  own  IoT  product  using  a  few  Lua  script  lines.

Multiple  GPIO  pins  on  the  board  allow  you  to  connect  the  board  with  other  peripherals  and  are  capable  of  generating  PWM,  I2C,  SPI,  and  UART  serial  communications.

- The  interface  of  the  module  is  mainly  divided  into  two  parts  including  both  Firmware  and  Hardware  where  former  runs  on  the  ESP8266  Wi-Fi  SoC  and  later  is  based  on  the  ESP-12  module.

The  firmware  is  based  on  Lua  –  A  scripting  language  that  is  easy  to  learn,  giving  a  simple  programming  environment  layered  with  a  fast  scripting  language  that  connects  you  with  a  well-known  developer  community.

And  open  source  firmware  gives  you  the  flexibility  to  edit,  modify  and  rebuilt  the  existing  module  and  keep  changing  the  entire  interface  until  you  succeed  in  optimizing  the  module  as  per  your  requirements.

- USB  to  UART  converter  is  added  on  the  module  that  helps  in  converting  USB  data  to  UART  data  which  mainly  understands  the  language  of  serial  communication.

Instead  of  the  regular  USB  port,  MicroUSB  port  is  included  in  the  module  that  connects  it  with  the  computer  for  dual  purposes:  programming  and  powering  up  the  board.

- The  board  incorporates  status  LED  that  blinks  and  turns  off  immediately,  giving  you  the  current  status  of  the  module  if  it  is  running  properly  when  connected  with  the  computer.

The  ability  of  module  to  establish  a  flawless  WiFi  connection  between  two  channels  makes  it  an  ideal  choice  for  incorporating  it  with  other  embedded  devices  like  Raspberry  Pi.

NodeMCU  Pinout:

NodeMCU  comes  with  a  number  of  GPIO  Pins.  Following  figure  shows  the  Pinout  of  the  board.

- There  is  a  candid  difference  between  VIN  and  VU  where  former  is  the  regulated  voltage  that  may  stand  somewhere  between  7  to  12  V  while  later  is  the  power  voltage  for  USB  that  must  be  kept  around  5  V.

Features:

- Open-source

- Arduino-like  hardware

- Status  LED

- MicroUSB  port

- Reset/Flash  buttons

- Interactive  and  Programmable

- Low  cost

- ESP8266  with  inbuilt  wifi

- USB  to  UART  converter

- GPIO  pins

As  mentioned  above,  a  cable  supporting  micro  USB  port  is  used  to  connect  the  board.  As  you  connect  the  board  with  a  computer,  LED  will  flash.  You  may  need  some  drivers  to  be  installed  on  your  computer  if  it  fails  to  detect  the  NodeMCU  board.  You  can  download  the  driver  from  this  page.

Note:  We  use  Arduino  IDE  software  for  programming  this  module.  It  is  important  to  note  that  the  pin  configuration  appearing  on  the  board  is  different  from  the  configuration  we  use  to  program  the  board  on  the  software  i.e.  when  we  write  code  for  targeting  pin  16  on  the  Arduino  IDE,  it  will  actually  help  is  laying  out  the  communication  with  the  D0  pin  on  the  module.

Following  figure  the  shows  the  pin  configuration  to  use  in  Arduino  IDE.

How  to  Power  NodeMCU

You  can  see  from  the  pinout  image  above,  there  are  five  ground  pins  and  three  3V3  pins  on  the  board.  The  board  can  be  powered  up  using  the  following  three  ways.

USB  Power.  It  proves  to  an  ideal  choice  for  loading  programs  unless  the  project  you  aim  to  design  requires  separate  interface  i.e.  disconnected  from  the  computer.

Provide  3.3V.  This  is  another  great  option  to  power  up  the  module.  If  you  have  your  own  off-board  regulator,  you  can  generate  an  instant  power  source  for  your  development  kit.

Power  Vin.  This  is  a  voltage  regulator  that  comes  with  the  ability  to  support  up  to  800  mA.  It  can  handle  somewhere  between  7  to  12  V.  You  cannot  power  the  devices  operating  at  3.3  V,  as  this  regulator  unable  to  generate  as  low  as  3.3V.

Applications:

NodeMCU  V3  is  mainly  used  in  the  WiFi  Applications  which  most  of  the  other  embedded  modules  fail  to  process  unless  incorporated  with  some  external  WiFi  protocol.  Following  are  some  major  applications  used  for  NodeMCU  V3.

- Internet  Smoked  Alarm

- VR  Tracker

- Octopod

- Serial  Port  Monitor

- ESP  Lamp

- Incubator  Controller

- IoT  home  automation

- Security  Alarms

3.4  Power  supply:

A  power  supply  is  a  component  that  provides  at  least  one  electrical  charge  with  power.  It  typically  converts  one  type  of  electrical  power  to  another,  but  it  can  also  convert  a  different  Energy  form  in  electrical  energy,  such  as  solar,  mechanical,  or  chemical.

A  power  supply  provides  electrical  power  to  components.  Usually  the  term  refers  to  devices  built  into  the  powered  component.  Computer  power  supplies,  for  example,  convert  AC  current  to  DC  current  and  are  generally  located  along  with  at  least  one  fan  at  the  back  of  the  computer  case.

Most  computer  power  supplies  also  have  an  input  voltage  switch  that,  depending  on  the  geographic  location,  can  be  set  to  110v/115v  or  220v/240v.  Due  to  the  different  power  voltages  supplied  by  power  outlets  in  different  countries,  this  switch  position  is  crucial.

Some  basic  components  used  in  the  supply  of  power:

3.5  Transformer:

A  transformer  is  a  static  electrical  gadget  that  exchanges  control  between  at  least  two  circuits.  A  fluctuating  current  creates  a  changing  attractive  motion  in  one  transformer  curl,  which  thus  actuates  a  differing  electromotive  power  over  a  second  loop  twisted  around  a  similar  center.

Without  a  metallic  association  between  the  two  circuits,  electrical  vitality  can  be  exchanged  between  the  two  loops.  The  enlistment  law  of  Faraday  found  in  1831  portrayed  the  impact  of  prompted  voltage  in  any  curl  because  of  the  changing  attractive  flux  surrounded  by  the  coil.

Circuit  of  transformer

Transformer

3.6  Rectifier:

A  rectifier  is  an  electrical  device  that  converts  alternating  current  (AC),  which  periodically  reverses  direction,  to  direct  current  (DC),  which  flows  in  only  one  direction.  The  process  is  known  as  rectification,  since  it  "straightens"  the  direction  of  current.

Rectifiers  have  many  uses,  but  are  often  found  to  serve  as  components  of  DC  power  supplies  and  direct  power  transmission  systems  with  high  voltage.  Rectification  can  be  used  in  roles  other  than  direct  current  generation  for  use  as  a  power  source.

Circuit  of  rectifier

Rectifier

3.7  Capacitors:

Capacitors  are  used  to  attain  from  the  connector  the  immaculate  and  smoothest  DC  voltage  in  which  the  rectifier  is  used  to  obtain  throbbing  DC  voltage  which  is  used  as  part  of  the  light  of  the  present  identity.  Capacitors  are  used  to  acquire  square  DC  from  the  current  AC  experience  of  the  current  channels  so  that  they  can  be  used  as  a  touch  of  parallel  yield.

Capacitor

3.8  Voltage  regulators:

The  78XX  voltage  controller  is  mainly  used  for  voltage  controllers  as  a  whole.  The  XX  speaks  to  the  voltage  delivered  to  the  specific  gadget  by  the  voltage  controller  as  the  yield.  7805  will  supply  and  control  5v  yield  voltage  and  12v  yield  voltage  will  be  created  by  7812.

The  voltage  controllers  are  that  their  yield  voltage  as  information  requires  no  less  than  2  volts.  For  example,  7805  as  sources  of  information  will  require  no  less  than  7V,  and  7812,  no  less  than  14  volts.  This  voltage  is  called  Dropout  Voltage,  which  should  be  given  to  voltage  controllers.

7805  voltage  regulator  with  pinout

7812  voltage  regulator  with  pinout

3.9  ESP32  AI  THINKER  BOARD:

The  ESP32-CAM  is  a  development  board  with  an  ESP32-S  chip,  an  OV2640  camera,  microSD  card  slot  and  several  GPIOs  to  connect  peripherals.

## Pinout  Diagram

The  following  image  shows  the  pinout  diagram  for  the  ESP32-CAM  AI-Thinker

## Schematic  Diagram

The  following  figure  shows  the  schematic  diagram  for  the  ESP32-CAM.

## Power  Pins

The  ESP32-CAM  comes  with  three  GND  pins  (colored  in  black  color)  and  two  power  pins  (colored  with  red  color):  3.3V  and  5V.

You  can  power  the  ESP32-CAM  through  the  3.3V  or  5V  pins.  However,  many  people  reported  errors  when  powering  the  ESP32-CAM  with  3.3V,  so  we  always  advise  to  power  the  ESP32-CAM  through  the  5V  pin.

### Power  output  pin

There’s  also  the  pin  labeled  on  the  silkscreen  as  VCC  (colored  with  a  yellow  rectangle).  You  should  not  use  that  pin  to  power  the  ESP32-CAM.  That  is  an  output  power  pin.  It  can  either  output  5V  or  3.3V.

In  our  case,  the  ESP32-CAM  outputs  3.3V  whether  it  is  powered  with  5V  or  3.3V.  Next  to  the  VCC  pin,  there  are  two  pads.  One  labeled  as  3.3V  and  other  as  5V.

If  you  look  closely,  you  should  have  a  jumper  on  the  3.3V  pads.  If  you  want  to  have  an  output  of  5V  on  the  VCC  pin,  you  need  to  unsolder  that  connection  and  solder  the  5V  pads.

## Serial  Pins

GPIO  1  and  GPIO  3  are  the  serial  pins  (TX  and  RX,  respectively).  Because  the  ESP32-CAM  doesn’t  have  a  built-in  programmer,  you  need  to  use  these  pins  to  communicate  with  the  board  and  upload  code.

The  best  way  to  upload  code  to  the  ESP32-CAM  is  using  an  FTDI  programmer.

Learn  how  to  upload  code  to  the  ESP32-CAM  AI-Thinker.

You  can  use  GPIO  1  and  GPIO  3  to  connect  other  peripherals  like  outputs  or  sensors  after  uploading  the  code.  However,  you  won’t  be  able  to  open  the  Serial  Monitor  and  see  if  everything  is  going  well  with  your  setup.

## GPIO  0

GPIO  0  determines  whether  the  ESP32  is  in  flashing  mode  or  not.  This  GPIO  is  internally  connected  to  a  pull-up  10k  Ohm  resistor.

When  GPIO  0  is  connected  to  GND,  the  ESP32  goes  into  flashing  mode  and  you  can  upload  code  to  the  board.

GPIO  0  connected  to  GND  »  ESP32-CAM  in  flashing  mode.To  make  the  ESP32  run  “normally”,  you  just  need  to  disconnect  GPIO  0  from  GND.

## MicroSD  Card  Connections

| MicroSD card | ESP32 |
| --- | --- |
| CLK | GPIO 14 |
| CMD | GPIO 15 |
| DATA0 | GPIO 2 |
| DATA1 / flashlight | GPIO 4 |
| DATA2 | GPIO 12 |
| DATA3 | GPIO 13 |

The  following  pins  are  used  to  interface  with  the  microSD  card  when  it  is  on  operation.

If  you’re  not  using  the  microSD  card,  you  can  use  these  pins  as  regular  inputs/outputs.  You  can  take  a  look  at  the  ESP32  pinout  guide  to  see  the  features  of  these  pins.

All  these  GPIOs  are  RTC  and  support  ADC:  GPIOs  2,  4,  12,  13,  14,  and  15.

## Flashlight  (GPIO  4)

The  ESP32-CAM  has  a  very  bright  built-in  LED  that  can  work  as  a  flash  when  taking
photos.  That  LED  is  internally  connected  to  GPIO  4.

That  GPIO  is  also  connected  to  the  microSD  card  slot,  so  you  may  have  troubles  when  trying  to  use  both  at  the  same  time  –  the  flashlight  will  light  up  when  using  the  microSD  card.

Note:  one  of  our  readers  shared  that  if  you  initialize  the  microSD  card  as  follows,  you  won’t  have  this  problem  because  the  microSD  card  won’t  use  that  data  line.*

SD_MMC.begin("/sdcard",  true)

*  we  found  that  this  works  and  that  the  LED  will  not  make  that  flash  effect.  However,  the  LED  remains  on  with  low  brightness  –  we’re  not  sure  if  we  are  missing  something.

## GPIO  33  –  Built-in  Red  LED

Next  to  the  RST  button,  there’s  an  on-board  red  LED.  That  LED  is  internally  connected  to  GPIO  33.  You  can  use  this  LED  to  indicate  that  something  is  happening.  For  example,  if  the  Wi-Fi  is  connected,  the  LED  is  red  or  vice-versa.

That  LED  works  with  inverted  logic,  so  you  send  a  LOW  signal  to  turn  it  on  and  a  HIGH  signal  to  turn  it  off.

## Camera  Connections

The  connections  between  the  camera  and  the  ESP32-CAM  AI-Thinker  are  shown  in  the  following  table.

| OV2640 CAMERA | ESP32 | Variable name in code |
| --- | --- | --- |
| D0 | GPIO 5 | Y2_GPIO_NUM |
| D1 | GPIO 18 | Y3_GPIO_NUM |
| D2 | GPIO 19 | Y4_GPIO_NUM |
| D3 | GPIO 21 | Y5_GPIO_NUM |
| D4 | GPIO 36 | Y6_GPIO_NUM |
| D5 | GPIO 39 | Y7_GPIO_NUM |
| D6 | GPIO 34 | Y8_GPIO_NUM |
| D7 | GPIO 35 | Y9_GPIO_NUM |
| XCLK | GPIO 0 | XCLK_GPIO_NUM |
| PCLK | GPIO 22 | PCLK_GPIO_NUM |
| VSYNC | GPIO 25 | VSYNC_GPIO_NUM |
| HREF | GPIO 23 | HREF_GPIO_NUM |
| SDA | GPIO 26 | SIOD_GPIO_NUM |
| SCL | GPIO 27 | SIOC_GPIO_NUM |
| POWER PIN | GPIO 32 | PWDN_GPIO_NUM |

3.10  Relay:

What  is  a  relay?

A  relay  is  an  electromagnetic  switch  that  is  used  to  turn  on  and  turn  off  a  circuit  by  a  low  power  signal,  or  where  several  circuits  must  be  controlled  by  one  signal.

Most  of  the  high  end  industrial  application  devices  have  relays  for  their  effective  working.  Relays  are  simple  switches  which  are  operated  both  electrically  and  mechanically.  Relays  consist  of  an  electromagnet  and  also  a  set  of  contacts.  The  switching  mechanism  is  carried  out  with  the  help  of  the  electromagnet.  There  are  also  other  operating  principles  for  its  working.  But  they  differ  according  to  their  applications.  Most  of  the  devices  have  the  application  of  relays.

### Pin  Diagram:

Why  is  a  relay  used?

The  main  operation  of  a  relay  comes  in  places  where  only  a  low-power  signal  can  be  used  to  control  a  circuit.  It  is  also  used  in  places  where  only  one  signal  can  be  used  to  control  a  lot  of  circuits.  The  application  of  relays  started  during  the  invention  of  telephones.  They  played  an  important  role  in  switching  calls  in  telephone  exchanges.  They  were  also  used  in  long  distance  telegraphy.  They  were  used  to  switch  the  signal  coming  from  one  source  to  another  destination.  After  the  invention  of  computers  they  were  also  used  to  perform  Boolean  and  other  logical  operations.  The  high  end  applications  of  relays  require  high  power  to  be  driven  by  electric  motors  and  so  on.  Such  relays  are  called  contactors.

### Relay  Design

- There  are  only  four  main  parts  in  a  relay.  They  are

- Electromagnet

- Movable  Armature

- Switch  point  contacts

- Spring

The  figures  given  below  show  the  actual  design  of  a  simple  relay.

Relay  Construction

It  is  an  electro-magnetic  relay  with  a  wire  coil,  surrounded  by  an  iron  core.  A  path  of  very  low  reluctance  for  the  magnetic  flux  is  provided  for  the  movable  armature  and  also  the  switch  point  contacts.

The  movable  armature  is  connected  to  the  yoke  which  is  mechanically  connected  to  the  switch  point  contacts.  These  parts  are  safely  held  with  the  help  of  a  spring.  The  spring  is  used  so  as  to  produce  an  air  gap  in  the  circuit  when  the  relay  becomes  de-energized.

### How  relay  works?

The  relay  function  can  be  better  understood  by  explaining  the  following  diagram  given  below.

Relay  Design

The  diagram  shows  an  inner  section  diagram  of  a  relay.  An  iron  core  is  surrounded  by  a  control  coil.  As  shown,  the  power  source  is  given  to  the  electromagnet  through  a  control  switch  and  through  contacts  to  the  load.  When  current  starts  flowing  through  the  control  coil,  the  electromagnet  starts  energizing  and  thus  intensifies  the  magnetic  field.  Thus  the  upper  contact  arm  starts  to  be  attracted  to  the  lower  fixed  arm  and  thus  closes  the  contacts  causing  a  short  circuit  for  the  power  to  the  load.  On  the  other  hand,  if  the  relay  was  already  de-energized  when  the  contacts  were  closed,  then  the  contact  move  oppositely  and  make  an  open  circuit.

As  soon  as  the  coil  current  is  off,  the  movable  armature  will  be  returned  by  a  force  back  to  its  initial  position.  This  force  will  be  almost  equal  to  half  the  strength  of  the  magnetic  force.  This  force  is  mainly  provided  by  two  factors.  They  are  the  spring  and  also  gravity.

Relays  are  mainly  made  for  two  basic  operations.  One  is  low  voltage  application  and  the  other  is  high  voltage.  For  low  voltage  applications,  more  preference  will  be  given  to  reduce  the  noise  of  the  whole  circuit.  For  high  voltage  applications,  they  are  mainly  designed  to  reduce  a  phenomenon  called  arcing.

### 3.10.1  Relay  Basics

The  basics  for  all  the  relays  are  the  same.  Take  a  look  at  a  4  pin  relay  shown  below.  There  are  two  colors  shown.  The  green  color  represents  the  control  circuit  and  the  red  color  represents  the  load  circuit.  A  small  control  coil  is  connected  onto  the  control  circuit.  A  switch  is  connected  to  the  load.  This  switch  is  controlled  by  the  coil  in  the  control  circuit.  Now  let  us  take  the  different  steps  that  occur  in  a  relay.

Relay  operation

- 3.10.2  Energized  Relay  (ON)

As  shown  in  the  circuit,  the  current  flowing  through  the  coils  represented  by  pins  1  and  3  causes  a  magnetic  field  to  be  aroused.  This  magnetic  field  causes  the  closing  of  the  pins  2  and  4.  Thus  the  switch  plays  an  important  role  in  the  relay  working.  As  it  is  a  part  of  the  load  circuit,  it  is  used  to  control  an  electrical  circuit  that  is  connected  to  it.  Thus,  when  the  electrical  relay  in  energized  the  current  flow  will  be  through  the  pins  2  and  4.

Energized  Relay  (ON)

- 3.10.3  De  –  Energized  Relay  (OFF)

As  soon  as  the  current  flow  stops  through  pins  1  and  3,  the  relay  switch  opens  and  thus  the  open  circuit  prevents  the  current  flow  through  pins  2  and  4.  Thus  the  relay  becomes  de-energized  and  thus  in  off  position.

De-Energized  Relay  (OFF)

In  simple,  when  a  voltage  is  applied  to  pin  1,  the  electromagnet  activates,  causing  a  magnetic  field  to  be  developed,  which  goes  on  to  close  the  pins  2  and  4  causing  a  closed  circuit.  When  there  is  no  voltage  on  pin  1,  there  will  be  no  electromagnetic  force  and  thus  no  magnetic  field.  Thus  the  switches  remain  open.

### 3.11  Pole  and  Throw

Relays  have  the  exact  working  of  a  switch.  So,  the  same  concept  is  also  applied.  A  relay  is  said  to  switch  one  or  more  poles.  Each  pole  has  contacts  that  can  be  thrown  in  mainly  three  ways.  They  are

- Normally  Open  Contact  (NO):  NO  contact  is  also  called  a  make  contact.  It  closes  the  circuit  when  the  relay  is  activated.  It  disconnects  the  circuit  when  the  relay  is  inactive.

- Normally  Closed  Contact  (NC):  NC  contact  is  also  known  as  break  contact.  This  is  opposite  to  the  NO  contact.  When  the  relay  is  activated,  the  circuit  disconnects.  When  the  relay  is  deactivated,  the  circuit  connects.

- Change-over  (CO)  /  Double-throw  (DT)  Contacts:  This  type  of  contacts  are  used  to  control  two  types  of  circuits.  They  are  used  to  control  a  NO  contact  and  also  a  NC  contact  with  a  common  terminal.  According  to  their  type  they  are  called  by  the  names  break  before  make  and  make  before  break  contacts.

Relays  can  be  used  to  control  several  circuits  by  just  one  signal.  A  relay  switches  one  or  more  poles,  each  of  whose  contacts  can  be  thrown  by  energizing  the  coil.

Relays  are  also  named  with  designations  like

- Single  Pole  Single  Throw  (SPST):  The  SPST  relay  has  a  total  of  four  terminals.  Out  of  these  two  terminals  can  be  connected  or  disconnected.  The  other  two  terminals  are  needed  for  the  coil  to  be  connected.

- Single  Pole  Double  Throw  (SPDT):  The  SPDT  relay  has  a  total  of  five  terminals.  Out  of  these  two  are  the  coil  terminals.  A  common  terminal  is  also  included  which  connects  to  either  of  two  others.

- Double  Pole  Single  Throw  (DPST):  The  DPST  relay  has  a  total  of  six  terminals.  These  terminals  are  further  divided  into  two  pairs.  Thus  they  can  act  as  two  SPST  which  are  actuated  by  a  single  coil.  Out  of  the  six  terminals  two  of  them  are  coil  terminals.

- Double  Pole  Double  Throw  (DPDT):  The  DPDT  relay  is  the  biggest  of  all.  It  has  mainly  eight  relay  terminals.  Out  of  these  two  rows  are  designed  to  be  change  over  terminals.  They  are  designed  to  act  as  two  SPDT  relays  which  are  actuated  by  a  single  coil.

### 3.11.1  Relay  Applications

- A  relay  circuit  is  used  to  realize  logic  functions.  They  play  a  very  important  role  in  providing  safety  critical  logic.

- Relays  are  used  to  provide  time  delay  functions.  They  are  used  to  time  the  delay  open  and  delay  close  of  contacts.

- Relays  are  used  to  control  high  voltage  circuits  with  the  help  of  low  voltage  signals.  Similarly  they  are  used  to  control  high  current  circuits  with  the  help  of  low  current  signals.

- They  are  also  used  as  protective  relays.  By  this  function  all  the  faults  during  transmission  and  reception  can  be  detected  and  isolated.

#### 3.11.2  Application  of  Overload  Relay

Overload  relay  is  an  electro-mechanical  device  that  is  used  to  safeguard  motors  from  overloads  and  power  failures.  Overload  relays  are  installed  in  motors  to  safeguard  against  sudden  current  spikes  that  may  damage  the  motor.  An  overload  relay  switch  works  in  characteristics  with  current  over  time  and  is  different  from  circuit  breakers  and  fuses,  where  a  sudden  trip  is  made  to  turn  off  the  motor.  The  most  widely  used  overload  relay  is  the  thermal  overload  relay  where  a  bimetallic  strip  is  used  to  turn  off  the  motor.  This  strip  is  set  to  make  contact  with  a  contactor  by  bending  itself  with  rising  temperatures  due  to  excess  current  flow.  The  contact  between  the  strip  and  the  contactor  causes  the  contactor  to  de-energize  and  restricts  the  power  to  the  motor,  and  thus  turns  it  off.

Another  type  of  overload  motor  is  the  electronic  type  which  continuously  watches  the  motor  current,  whereas  the  thermal  overload  relay  shuts  off  the  motor  depending  on  the  rise  of  temperature/heat  of  the  strip.

All  overload  relays  available  to  buy  comes  in  different  specifications,  the  most  important  of  them  being  the  current  ranges  and  response  time.  Most  of  them  are  designed  to  automatically  reset  to  work  after  the  motor  is  turned  back  on.

### 3.11.3  Relay  Selection

You  must  note  some  factors  while  selecting  a  particular  relay.  They  are

- Protection  Different  protections  like  contact  protection  and  coil  protection  must  be  noted.  Contact  protection  helps  in  reducing  arcing  in  circuits  using  inductors.  Â  Coil  protection  helps  in  reducing  surge  voltage  produced  during  switching.

- Look  for  a  standard  relay  with  all  regulatory  approvals.

- Switching  time  Ask  for  high  speed  switching  relays  if  you  want  one.

- Ratings  There  are  current  as  well  as  voltage  ratings.  The  current  ratings  vary  from  a  few  amperes  to  about  3000  amperes.  Â  In  case  of  voltage  ratings,  they  vary  from  300  Volt  AC  to  600  Volt  AC.  There  are  also  high  voltage  relays  of  about  15,000  Volts.

- Type  of  contact  used  whether  it  is  a  NC  or  NO  or  closed  contact.

- Select  Make  before  Break  or  Break  before  Make  contacts  wisely.

- Isolation  between  coil  circuit  and  contacts

## CHAPTER  4

## SOFTWARE  REQUIREMENT

4.1  Adafruit:

Adafruit  Industries  is  an  open-source  hardware  company  based  in  New  York  City.  It  was  founded  by  Limor  Fried  in  2005.  The  company  designs,  manufactures  and  sells  a  number  of  electronics  products,  electronics  components,  tools  and  accessories.  It  also  produces  a  number  of  learning  resources,  including  live  and  recorded  videos  related  to  electronics,  technology,  and  programming.  MQTT,  or  message  queue  telemetry  transport,  is  a  protocol  for  device  communication  that  Adafruit  IO  supports.  Using  a  MQTT  library  or  client  you  can  publish  and  subscribe  to  a  feed  to  send  and  receive  feed  data.

# 4.2  Adafruit  IO  Steps

First,  created  account  at  www.Adafruit.io

Now,  create  dashboard  at  Adafruit.  This  dashboard  is  a  user  interface  to  control  things  remotely.

After  following  above  steps,  provide  name  to  the  dashboard  and  save  it.  We  can  see  our  dashboard  as  follows,

Now,  create  feed  (user  interface)  to  control  light  On-Off.  To  create  it,  just  click  on  ‘+’  symbol  and  select  toggle  feed  shown  below,

After  selecting  toggle  feed,  pop-up  window  appears  as  shown  below.

Enter  name  of  our  feed  (shown  in  red  box)  and  create  it.  After  creation,  select  the  created  feed  (here  mine  is  light)  and  then  click  on  Next  step.

In  the  next  step  configure  the  feed  which  is  shown  below,

Here,  I  used  0(OFF)  and  1(ON)  text  for  button  and  then  click  on  create.  This  will  create  toggle  button  on  your  dashboard  which  can  be  used  to  control  things  remotely.

4.3  IFTTT:

IFTTT  (If  This  Then  That)  If  This  Then  That,  also  known  as  IFTTT  is  a  free  web-based  service  to  create  chains  of  simple  conditional  statements,  called  applets.  An  applet  is  triggered  by  changes  that  occur  within  other  web  services  such  as  Gmail,  Facebook,  Telegram,  Instagram,  or  Pinterest.  For  example,  an  applet  may  send  an  e-mail  message  if  the  user  tweets  using  a  hashtag  or  copy  a  photo  on  Facebook  to  a  user's  archive  if  someone  tags  a  user  in  a  photo.  Here,  IFTTT  is  used  to  use  google  assistant  service  and  Adafruit  service  in  chain.  So,  Google  assistant  is  used  to  control  light  of  my  home  by  saying  Ok  google,  turn  the  light  ON  or  OFF.  Then  IFTTT  interpret  the  message  and  can  send  it  to  Adafruit’s  dashboard  as  an  understandable  command  to  the  created  feed.

First  step  is  creating  account  on  IFTTT.

Note:  Create  account  on  IFTTT  by  using  same  e-mail  id  which  have  been  used  for  Adafruit.

After  account  creation,  click  on  My  Applets  and  then  select  New  Applet  shown  below,

After  selecting  a  new  applet,  we  get  a  new  page  in  which  we  should  click  on  to  This

Then  search  for  Google  Assistant  and  select  it.

Searching  Google  assistant  Now,  enter  voice  phrases  which  will  be  used  as  a  command  for  google  assistant.

Any  phrase  can  be  entered  as  per  the  application.  The  phrases  entered  in  the  showing  trigger  fields  is  for  making  Light  ON.  For  making  Light  OFF,  another  applet  with  different  phrases  have  to  be  created.  Now,  another  page  will  be  shown  as  shown  in  Fig  3.13  in  which  user  have  to  click  on  that  option  which  is  used  to  connect  Google  Assistant  with  Adafruit.

Then  search  for  Adafruit  and  select  it.

After  selecting  Adafruit

Now  enter  what  data  needed  to  send  to  which  feed  of  Adafruit  dashboard.

Click  on  Create  Action.  So,  when  Google  Assistant  is  used  on  my  mobile.  This  will  trigger  the  event  on  Adafruit  dashboard  which  is  continuously  monitored  by  the  microcontroller  (here  NodeMCU).  This  microcontroller  will  take  action  as  per  the  data  change  on  the  Adafruit  dashboard.

ADVANTAGES

- Portable

- Low  cost

- Easy  to  use

APPLICATIONS

- Home  automation  –  This  project  can  be  used  to  control  various  Home  Appliances

- We  can  control  device  from  a  long  distance;  thus,  it  gives  ease  of  access.

- Faster  operation  and  efficient.

- No  need  to  carry  separate  remote  or  any  other  controlling  unit.

## CHAPTER  5

## RESULTS/FUTURE SCOPE

Circuit  Connection  Diagram

When  system  detects  the  authorized  face,  it  shows  like  the  “Hello  subject  0”.

When  system  detects  the  unauthorized  face  it  will  give  Intruder  Alert.

CHAPTER  6

## CONCLUSION & FUTURE SCOPE

By  the  end  of  successful  completion  of  this  work  it  was  concluded  that  through  IOT  we  can  connect  multiple  input/  output  devices,  multiple  sensors  and  actuators  in  a  network  so  that  they  can  talk  to  each  other,  data  acquired  from  these  things  can  be  used  to  keep  a  log  or  monitor  or  control  the  other  things  without  human  intervention  and  much  more.  Thus,  IOT  is  like  global  networks  which  give  the  communication  between  things  to  things,  human  to  things  and  human  to  human.  IOT  is  the  development  of  existing  internet  facility  to  manage  everything  which  exists  in  the  world  or  exists  in  the  future.

FUTURE SCOPE :

The various future applications may be used by controlling various household devices of house with internet.In the future work, the persons other than the residents will also be considered. o Industrial automation and management through internet.Improvement of security problems Used in extremely restricted areas

REFERENCES

[1]  M.  Daneshvar,  M.  Pesaran,  and  B.  Mohammadi-Ivatloo,  ‘‘Transactive

energy  in  future  smart  homes,’’  in  The  Energy  Internet.  Amsterdam,

The  Netherlands:  Elsevier,  2019,  pp.  153–179.

[2]  R.  Sinha,  S.  Patil,  L.  Gomes,  and  V.  Vyatkin,  ‘‘A  survey  of  static  formal

methods  for  building  dependable  industrial  automation  systems,’’  IEEE

Trans.  Ind.  Informat.,  vol.  15,  no.  7,  pp.  3772–3783,  Jul.  2019.

[3]  W.  A.  Jabbar,  M.  H.  Alsibai,  N.  S.  S.  Amran,  and  S.  K.  Mahayadin,  ‘‘Design

and  implementation  of  IoT-based  automation  system  for  smart  home,’’  in

Proc.  Int.  Symp.  Netw.,  Comput.  Commun.(ISNCC),  Jun.  2018,  pp.  1–6.

[4]  H.  Ning,  F.  Shi,  T.  Zhu,  Q.  Li,  and  L.  Chen,  ‘‘A  novel  ontology  consistent

with  acknowledged  standards  in  smart  homes,’’  Comput.  Netw.,  vol.  148,

pp.  101–107,  Jan.  2019.

[5] W. Li, T. Logenthiran, V.-T. Phan, and W. L. Woo, ‘‘A novel smart energy

theft system (SETS) for IoT-based smart home,’’ IEEE Internet Things J.,

vol. 6, no. 3, pp. 5531–5539, Jun. 2019.

[6] W. A. Jabbar, W. K. Saad, and M. Ismail, ‘‘MEQSA-OLSRv2:

A multicriteria-based hybrid multipath protocol for energy-efficient and

QoS-aware data routing in MANET-WSN convergence scenarios of IoT,’’

IEEE Access, vol. 6, pp. 76546–76572, 2018.

[7] D. Anandhavalli, N. S. Mubina, and P. Bharathi, ‘‘Smart home automation

control using Bluetooth and GSM,’’ Int. J. Informative Futuristic Res.,

vol. 2, no. 8, pp. 2547–2552, 2015.

[8] B. Davidovic and A. Labus, ‘‘A smart home system based on sensor

technology,’’ Facta Universitatis, Ser., Electron. Energetics, vol. 29, no. 3,

pp. 451–460, 2015.

[9] N. David, A. Chima, A. Ugochukwu, and E. Obinna, ‘‘Design of a home

automation system using arduino,’’ Int. J. Sci. Eng. Res., vol. 6, no. 6,

pp. 795–801, 2015.

[10] R. K. Kodali and S. Soratkal, ‘‘MQTT based home automation system

using ESP8266,’’ in Proc. IEEE Region 10 Humanitarian Technol. Conf.

(R10-HTC), Dec. 2016, pp. 1–5.

[11] A. ElShafee and K. A. Hamed, ‘‘Design and implementation of a WIFI

based home automation system,’’ World Acad. Sci., Eng. Technol., vol. 68,

pp. 2177–2180, Aug. 2012.